Practice: 51 MCU Simulated Industrial RTU Register Tampering

Overview

This project simulates a tampering attack on an industrial RTU's holding register using a 51 microcontroller and an AT24C02 EEPROM. The device exposes an unprotected UART debug interface that allows reading and modifying the register value without authentication.

Attack chain:

- Physical access to UART debug port

- Send SETcommand to modify register value

- Logic analyzer captures I2C bus traffic during EEPROM write

- (Future) Offline EEPROM modification via CH341A programmer

Hardware

- 51 MCU board (STC89C52RC @ 11.0592MHz)

- AT24C02 I2C EEPROM (stores 4-byte register at address 0x00-0x03)

- CH340 USB-to-Serial adapter

- 24MHz 8-channel logic analyzer (FX2 clone)

Software

- Keil C51 (MCU firmware)

- PulseView (logic analyzer)

- Python 3 + pyserial (attack script)

Implementation Details

MCU Firmware code/main.c

- Initialization: On boot, reads 4-byte register from EEPROM address 0x00-0x03

- Normal operation: Button press increments register by 1, writes back to EEPROM

- UART command parsing: Uses timeout-based detection (1 second idle = end of command)

- READ: Sends 4 bytes (big-endian) via UART

- SET XXXX: Parses next 4 bytes as ASCII digits, adds 0x30 offset, writes to EEPROM

- Other input: Clears buffer, resets state machine

## Logic Analyzer Capture

- UART: Captured continuous 4-byte READ response 0x310x320x330x34="1234"

- I2C: Captured EEPROM write frame (Device 0x50, Addr 0x00, Data 0x31 0x32 0x33 0x34)

## Key Challenges

TI flag race condition:Interrupt service routine's if(TI){ TI=0; }interfered with main loop's while(!TI)during consecutive Uart_SendBytecalls. Solved by disabling serial interrupt ES=0 during multi-byte transmission.

Timeout threshold tuning:1000ms proved too long for rapid command sequences; reduced to 200ms for better responsiveness.

Corresponding Real-World Scenarios

| This Practice | Industrial Context |
| --- | --- |
| AT24C02 stores 4-byte register | PLC holding register / RTU configuration EEPROM |
| UART SETcommand without password | Debug port without authentication (CVE-2019-10945 etc.) |
| Logic analyzer captures I2C bus | Non-invasive bus sniffing / forensic analysis |
| Offline EEPROM modification (planned) | Configuration tampering (similar to Stuxnet approach) |

Next Steps

- Implement offline EEPROM modification using CH341A programmer

- Port to STM32 with hardware I2C and Modbus RTU protocol

- Add CRC checksum to prevent simple tampering (defense perspective)